AMERICAN INTERNATIONAL

UNIVERSITY-BANGLADESH

408/1, Kuratoli, Khilkhet, Dhaka 1229, Bangladesh

Title: Familiarization with microcontroller, study of blink test using and implementation of a traffic control system using microcontrollers

| Lab report no: 01 |  | Date of Submission: 25-09-2023 |
| --- | --- | --- |
| Course Title: Microprocessor & Embedded System |  |  |
| Course Code: |  | Section: L |
| Semester: 09 | 2023-24 | Course Teacher: PROTIK PARVEZ SHEIKH |

Declaration and Statement of Authorship:

- I/we hold a copy of this Assignment/Case-Study, which can be produced if the original is lost/damaged.

- This Assignment/Case-Study is my/our original work and no part of it has been copied from any other student’s work or from any other source except where due acknowledgement is made.

- No part of this Assignment/Case-Study has been written for me/us by any other person except where such collaborationhas been authorized by the concerned teacher and is clearly acknowledged in the assignment.

- I/we have not previously submitted or currently submitting this work for any other course/unit.

- This work may be reproduced, communicated, compared and archived for the purpose of detecting plagiarism.

- I/we give permission for a copy of my/our marked work to be retained by the Faculty for review and comparison, including review by external examiners.

*  Student(s) must complete all details except the faculty use part.

** Please submit all assignments to your course teacher or the office of the concerned teacher.

Group Name

/

No.

:

| No | Name | ID | Program | Signature |
| --- | --- | --- | --- | --- |
| 1 | NOKIBUL ARFIN SIAM | 21-44793-1 | CSE | Siam |
| 2 | MD. IMRAN AHMED | 20-43738-2 | CSE | Imran |
| 3 | TANVIR HASAN TAMAL | 21-44626-1 | CSE | Tamal |
| 4 | TAZUDDIN AHMAD | 20-42787-1 | CSE | Tazuddin |
| 5 | MD. ZAMIUL SADIK NAHIN | 20-44228-3 | CSE | Nahin |

| Faculty use only |  |  |
| --- | --- | --- |
| FACULTYCOMMENTS | Marks Obtained |  |
|  | Total Marks |  |

Assignment/Case-Study Cover; © AIUB-2020

Title: Familiarization with microcontroller, study of blink test using and implementation of a traffic control system using microcontrollers

Introduction:

The objectives of this experiment are to-

1. Get familiar with Arduino microcontrollers.

2. Use an Arduino and delay functions to make an LED blink.

3. Implement an LED traffic control system using Arduino.

4. Simulate the microcontroller-based systems using proteus.

Equipment List:

- Arduino IDE (2.0.1 or any recent version)

- Arduino Microcontroller board

- Bread board

- LED lights (Red, Green, and Yellow)

- Three 200  resistors

- Jumper wires

Circuit diagram:

Fig-1: LED Blink Test using an Arduino Microcontroller Board

Fig-2: Traffic Control System using an Arduino Microcontroller Board

Code/program:

LED Blink

int led=13; void setup () { pinMode(led, OUTPUT); } void loop () { digitalWrite (led, HIGH); delay (1000); digitalWrite (led, LOW); delay (1000); }

Traffic Control System

#define RED_PIN 8 #define YELLOW_PIN 10 #define GREEN_PIN 12 int red_on = 3000; int red_yellow_on = 1000; int green_on = 3000; int green_blink = 500; int yellow_on = 1000; void setup() { pinMode(RED_PIN, OUTPUT); pinMode(YELLOW_PIN, OUTPUT); pinMode(GREEN_PIN, OUTPUT); } void loop() { digitalWrite(RED_PIN, HIGH); //to make red LED on delay(red_on); //to turn yellow LED on digitalWrite(YELLOW_PIN, HIGH); delay(red_yellow_on); //turning off RED_PIN and YELLOW_PIN, and turrning on greenLEd digitalWrite(RED_PIN, LOW); digitalWrite(YELLOW_PIN, LOW); digitalWrite(GREEN_PIN, HIGH); delay(green_on); digitalWrite(GREEN_PIN, LOW); //for turning green Led on and off for 3 times for(int i = 0; i < 3; i = i+1) { delay(green_blink); digitalWrite(GREEN_PIN, HIGH); delay(green_blink); digitalWrite(GREEN_PIN, LOW); } //for turning on yellow LED digitalWrite(YELLOW_PIN, HIGH); delay(yellow_on); digitalWrite(YELLOW_PIN, LOW); }

Hardware Implementation:

LED Blink Test

Traffic Control System

Simulation:

LED blink

Traffic Light

Flowchart

Discussion: The purpose of the experiment was to gain experience with the Arduino IDE software and to create an LED blink using the Arduino platform and its delay functions. Additionally, a traffic control system was built using the Arduino microcontroller. To begin, the code was written in the IDE software and tested on a breadboard circuit. Once confirmed, the code was then transferred to the Arduino board. The experiment was successfully completed without any hardware or code-related issues and produced similar results both in simulation and in real-life testing.

Reference(s):

1) https://www.arduino.cc/.

2) https://www.coursera.org/learn/arduino/lecture/ei4ni/1-10-first-glance-at-a-program

3) Jeremy Blue; Exploring Arduino: Tools and Techniques for Engineering Wizardry